EcoSweep: An Edge-AI Based Autonomous Garbage Detection and Collection Platform for Environmental Cleanup and Sanitation Assistance

Simran Amit Singh 
Bachelor of Science in Information Technology 
Vidyalankar School of Information Technology 
Mumbai, India 
Simransingh5414@gmail.com

Farhan Faizan Sayed 
Bachelor of Science in Information Technology 
Vidyalankar School of Information Technology 
Mumbai, India 
Farhansayed54@gmail.com

Abstract—Garbage cleanup across diverse environments, including indoor spaces, beaches, swampy areas, gutters, and manholes, remains largely manual, exposing workers to hygiene risks, disease transmission, and hazardous conditions. Existing robotic solutions are either expensive, depend on cloud-based processing, or operate only in structured settings. This paper presents EcoSweep, a low-cost autonomous robotic platform for garbage detection and collection in unstructured environments using edge AI. The system employs YOLOv8 Nano on a Raspberry Pi 4 for real-time object detection, uses bounding-box area as a distance proxy when ultrasonic sensors are unreliable, and implements a five-state autonomy logic (SEARCH, APPROACH_FAR, APPROACH_CLOSE, PICKUP, RECOVER) for robust navigation and pickup. A 5-axis robotic arm with gripper executes the pickup sequence. Experiments demonstrate detection accuracy above 85 percent for common garbage classes at 10 to 15 FPS on edge hardware, with successful pickup rates of approximately 80 percent in controlled indoor conditions as initial validation. The system operates entirely onboard without cloud dependency, achieving a total hardware cost significantly lower than commercial alternatives. EcoSweep demonstrates the feasibility of affordable, autonomous garbage collection using edge AI and vision-based control. The modular design is extensible to beach cleaning, gutter clearance, swampy-area cleanup, and manhole inspection, reducing human exposure to disease vectors and hazardous environments.

Keywords—Autonomous Robotics, Edge AI, Garbage Detection, YOLOv8, Object Detection, Raspberry Pi, Mobile Robot, Environmental Cleanup, Sanitation Assistance, Beach Cleaning.

# I. INTRODUCTION

## A. Background

Garbage and waste accumulation poses challenges across a wide spectrum of environments. Indoor spaces, homes, offices, and hospitals rely heavily on manual collection; in healthcare and industrial facilities, such work exposes workers to hygiene risks and potential disease transmission. Outdoor and semi-aquatic settings—beaches, wetlands, swampy areas—suffer from plastic pollution and debris that harms ecosystems and public health. Gutter and drainage systems accumulate trash that causes blockages, flooding, and breeding grounds for disease vectors. Manhole inspection and cleaning expose sanitation workers to confined spaces, toxic gases (H2S, methane), and physical hazards from solid waste and sludge; manual manhole cleaning is one of the most dangerous sanitation jobs globally. In each context, reducing direct human exposure while improving cleanup efficiency can protect workers from disease, injury, and hazardous conditions. The automation of garbage detection and collection, and robotic assistance in these diverse sanitation tasks, has therefore attracted interest from both industry and academia [1], [2].

## B. Problem Statement

Despite advances in mobile robotics and computer vision, affordable autonomous systems capable of detecting, approaching, and physically collecting garbage across unstructured environments (indoor, outdoor, beaches, gutters, swampy areas, and manhole-adjacent surfaces) remain scarce. Commercial cleaning robots such as the Roomba focus on vacuuming and mapping rather than discrete object pickup [3]. Research prototypes often depend on cloud-based inference [4], depth cameras [5], or high-end compute platforms, limiting their cost-effectiveness and deployment in resource-constrained or remote settings. Meanwhile, sanitation workers in hazardous roles lack low-cost robotic tools to reduce their exposure to disease and injury.

## C. Limitations of Current Systems

Current systems face several limitations. Cost: commercial or research-grade robots with manipulation capabilities typically cost thousands of dollars. Cloud dependency: cloud-based object detection introduces latency, requires connectivity, and raises privacy concerns. Structured environments: many solutions assume known layouts, fiducial markers, or controlled lighting. Depth sensors: RGB-D cameras add cost and may perform poorly in sunlight or on reflective surfaces.

## D. Research Gap

A gap exists for an affordable, autonomous, edge-AI based garbage collection platform that: (1) runs object detection entirely on low-cost edge hardware; (2) operates in unstructured environments (indoor, outdoor, beaches, gutters, swampy areas, manhole-adjacent) without prior mapping; (3) uses monocular vision and optional ultrasonic sensors for proximity, avoiding expensive depth cameras; (4) integrates perception, navigation, and manipulation in a single pipeline; (5) reduces human exposure to disease vectors and hazardous conditions across diverse sanitation contexts.

## E. Contributions

This paper proposes EcoSweep, an autonomous robotic system designed to address these challenges. The main contributions are: (1) edge-AI detection pipeline with real-time YOLOv8 Nano inference on Raspberry Pi 4; (2) bounding-box area as distance proxy when ultrasonic sensors are unreliable; (3) structured autonomy state machine (SEARCH, APPROACH_FAR, APPROACH_CLOSE, PICKUP, RECOVER); (4) robust pickup strategy with stability check and lost-detection fallback; (5) integrated system design from camera input to arm actuation; (6) platform extensibility to beach cleaning, gutter clearance, swampy-area cleanup, and manhole inspection.

## F. Paper Organization

The remainder of this paper is organized as follows. Section II reviews related work. Section III describes the system architecture. Section IV presents the methodology. Section V details the implementation. Section VI describes the experimental setup and results. Section VII discusses limitations, extended applications, and future work. Section VIII concludes the paper.

# II. RELATED WORK

## A. Autonomous Cleaning Robots

Commercial autonomous cleaning robots such as the iRobot Roomba [3] focus on vacuum-based floor cleaning and use structured navigation. They do not perform discrete object detection or manipulation. Industrial waste-sorting systems [6] use conveyor belts and fixed cameras, which are unsuitable for mobile indoor cleanup.

## B. Vision-Based Waste Detection

Mittal et al. [7] used CNNs for trash classification. Yu et al. [8] applied YOLOv3 for marine debris detection. Bircher et al. [4] proposed a garbage-collection robot but relied on cloud processing for inference.

## C. YOLO in Robotics and Edge Deployment

YOLO [9] and its variants are widely used for real-time object detection. YOLOv8 [10] offers model sizes suitable for edge deployment. Prior work has deployed YOLO on Raspberry Pi [11], [12], but integration with autonomous manipulation and robust pickup under partial detection loss remains underexplored.

## D. Mobile Robot Control and State Machines

State-machine-based control is common in mobile robotics [13]. Proportional control for steering is well established [14]. EcoSweep extends these ideas by combining distance-staged states with bbox-based distance estimation.

## E. Position of This Work

EcoSweep performs onboard inference, integrates perception with manipulation, uses bbox-area as a distance proxy when depth or ultrasonic data is unreliable, and implements robust pickup under partial visibility via a structured state machine. The platform targets low-cost edge hardware and diverse environments.

# III. SYSTEM ARCHITECTURE

## A. Overall Design

EcoSweep comprises three main subsystems: (1) a mobile app for user control and monitoring, (2) a Raspberry Pi serving as the AI and bridge node, and (3) an Arduino-based actuator and sensor controller. The mobile app communicates via Bluetooth SPP; the Pi runs YOLO detection and the bridge script; the Arduino controls motors, servos, and reads sensors. Figure 1 shows the system architecture diagram.

[INSERT FIG. 1: UML/system block diagram. Source: docs/diagrams/ecosweep-fig1-system-architecture.puml]

Fig. 1. EcoSweep system architecture: Flutter App, Bluetooth SPP, Raspberry Pi (YOLO, Bridge, Autonomy), USB Serial, Arduino Mega (motors, servos, sensors).

## B. Hardware Architecture

Table I summarizes the hardware components.

TABLE I. HARDWARE SPECIFICATIONS

| Component | Specification |
| --- | --- |
| Compute (AI) | Raspberry Pi 4 |
| Microcontroller | Arduino Mega 2560 |
| Camera | USB webcam, 640×480 @ 20 FPS |
| Drive | 4 DC motors, 2× BTS7960 (43A H-bridge) |
| Arm | 5 servos (base, arm, forearm, wrist, gripper), PCA9685 PWM driver |
| Ultrasonic | 3× HC-SR04 (front, left, right) |
| IMU | Adafruit MPU6050 (6-DOF) |
| Compass | QMC5883L |
| Connectivity | Bluetooth Classic SPP (RFCOMM channel 1) |

[INSERT FIG. 2: Hardware assembly photo or labeled diagram showing key components]

Fig. 2. EcoSweep robot prototype: chassis with camera, 5-axis arm, gripper, wheels, ultrasonic sensors.

## C. Software Architecture

The Raspberry Pi runs: (1) YOLO detection script—captures frames, runs YOLOv8 inference, writes detection to /tmp/ecosweep_detection.json, serves MJPEG stream at 15 FPS; (2) Bridge script—Bluetooth SPP server, Arduino serial bridge, and autonomy loop; (3) Flutter app—joystick control, autonomous mode toggle, FPV video, telemetry. The Arduino executes motor and servo commands and reports telemetry at 4 Hz.

## D. Communication Protocol

Commands: M:speed,turn (proportional drive); SA:ARM_DOWN_START, SA:GRIP_CLOSE_START, etc. (servo actions); MODE:AUTO_ON/OFF. Arduino responds with DATA:SENSORS:front,left,right.

## E. Sensor Fusion

EcoSweep fuses camera-based vision with ultrasonic data. When ultrasonic readings are reliable, front < 18 cm indicates arm reach. When ultrasonic is noisy or unavailable, the system uses bounding-box area as a distance proxy: larger bbox_area indicates a closer object.

# IV. METHODOLOGY

## A. Object Detection

We use YOLOv8 Nano (yolov8n.pt) from Ultralytics for real-time inference on Raspberry Pi 4. Garbage classes (COCO-derived): bottle, cup, cell phone, book, banana, apple, paper. Obstacle classes (safety): chair, couch, bed, dining table, potted plant. Parameters: input 416×416, confidence threshold 0.35, inference every 2nd frame to balance CPU load. Output includes decision (MOVE_LEFT, MOVE_RIGHT, CENTERED), confidence, bbox_area, bbox_center_x, person_detected, obstacle_detected. All fields are written to /tmp/ecosweep_detection.json for the autonomy loop.

[INSERT FIG. 3: Screenshot from FPV stream or detection visualization]

Fig. 3. Sample YOLO detection output with bounding boxes and class labels on 640×480 frame (bottle, cup detected).

## B. Distance Estimation

Stages: FAR (bbox_area < 40,000), APPROACH_CLOSE (40,000–55,000), PICKUP (≥ 55,000). Ultrasonic front < 18 cm also qualifies when available.

## C. Autonomy State Machine

The autonomy loop runs at 10 Hz and implements a five-state machine. Transitions are driven by bbox_area, frame-center alignment, and detection stability. Figure 4 shows the state diagram.

[INSERT FIG. 4: UML state diagram. Source: docs/diagrams/ecosweep-fig4-state-machine.puml]

Fig. 4. Autonomy state machine: SEARCH, APPROACH_FAR, APPROACH_CLOSE, PICKUP, RECOVER with transition conditions.

TABLE II. STATE DEFINITIONS AND BEHAVIORS

| State | Condition | Behavior |
| --- | --- | --- |
| SEARCH | No garbage | Rotate slowly; flip direction every 1.2 s |
| APPROACH_FAR | bbox < 40k | Drive forward; turn if off-center > 90 px |
| APPROACH_CLOSE | 40k ≤ bbox < 55k | Speed 40; proportional steering |
| PICKUP | bbox ≥ 55k, centered, stable 5 frames | Creep, arm down, grip, arm up |
| RECOVER | After pickup | Back up, turn, return to SEARCH |

TABLE III. AUTONOMY PARAMETERS

| Parameter | Value | Description |
| --- | --- | --- |
| FAR_BBOX | 40,000 | Threshold for APPROACH_FAR |
| PICKUP_BBOX_MIN | 55,000 | Minimum bbox for pickup trigger |
| PICKUP_STABLE_FRAMES | 5 | Frames conditions must hold before pickup |
| CENTER_MARGIN_PX | 35 | CENTERED if within ±35 px of frame center |
| GARBAGE_MIN_CONF | 0.35 | Minimum confidence for valid detection |
| NO_DETECTION_TIMEOUT_S | 2.0 | Return to SEARCH after timeout |

## D. Pickup Strategy

Stability check: 5 consecutive frames with bbox ≥ 55k, CENTERED, front < 18 cm. Lost-detection fallback: if last_centered_bbox ≥ 38k and time < 0.8 s, proceed to PICKUP. Pickup sequence: stop → creep 0.4 s → arm down → grip close 1.8 s → arm up → RECOVER.

## E. Safety

Person detection: stop. Obstacle (optional): furniture triggers STOP. Ultrasonic (optional): front < 15 cm triggers STOP. Motor watchdog: Arduino stops if no M: for 500 ms.

# V. IMPLEMENTATION

## A. Hardware Setup

The robot chassis supports four driven wheels via two BTS7960 motor drivers. The 5-axis arm uses standard servos driven by a PCA9685 over I2C. Ultrasonic sensors (HC-SR04) are mounted front, left, and right. A USB webcam (640×480 @ 20 FPS) is fixed at the front. The Raspberry Pi and Arduino connect via USB; the Pi runs the bridge and YOLO script; the Android app pairs via Bluetooth SPP.

[INSERT FIG. 5: Close-up photos of critical hardware components]

Fig. 5. Key components: USB camera, ultrasonic sensors (front/left/right), 5-axis arm with gripper, drive motors.

## B. Software Pipeline and Data Flow

The YOLO script uses separate threads for (1) frame capture at camera FPS, (2) YOLO inference every 2nd frame, and (3) Flask MJPEG streaming. This avoids blocking the stream on inference. The bridge runs the autonomy loop in a daemon thread; the main thread handles Bluetooth I/O and Arduino forwarding. Figure 6 illustrates the data flow from camera to pickup command.

[INSERT FIG. 6: Sequence or data flow diagram. Source: docs/diagrams/ecosweep-data-flow.puml]

Fig. 6. Data flow: Camera to YOLO thread to detection file; autonomy loop reads file, sends M:/SA: via bridge to Arduino.

## C. Software Stack

Raspberry Pi: Python 3, OpenCV, Ultralytics YOLOv8, Flask, PySerial, PyBluez. Arduino: Adafruit PWM Servo Driver, MPU6050, QMC5883L. App: Flutter, flutter_bluetooth_serial, mjpeg_stream for FPV.

## D. Challenges and Solutions

TABLE: CHALLENGES AND SOLUTIONS

| Challenge | Solution |
| --- | --- |
| Ultrasonic noise | Bbox-area fallback; optional disable of ultrasonic check |
| Detection loss near gripper | Lost-detection logic with last_centered_bbox threshold |
| Jerky search behavior | Sustained turn for 1.2 s before flipping direction |
| App–Pi–Arduino coordination | MODE:AUTO_ON/OFF; autonomy owns M: when AUTO_ON |

# VI. EXPERIMENTAL SETUP AND RESULTS

## A. Experimental Setup

Environment: indoor room, mixed lighting, flat floor. Test objects: plastic bottles, cups, books. Hardware: Raspberry Pi 4, USB webcam 640×480, Arduino Mega 2560. Software: Ultralytics YOLOv8, Python 3, OpenCV, Flask. Frame rate: 15 FPS stream; YOLO runs every 2nd frame (~7–8 detections/s). Trials: approach-and-pickup runs per scenario.

[INSERT FIG. 7: Photo of experimental setup]

Fig. 7. Test environment: indoor room with robot, sample objects (bottles, cups, books), and lighting setup.

[INSERT FIG. 8: Android app interface screenshots]

Fig. 8. Mobile app screenshots: FPV video stream, joystick control, autonomous mode toggle, telemetry display.

## B. Metrics

Detection success: fraction of frames with correct garbage detection when object in view. Pickup success: fraction of trials ending in successful grip and lift. FPS: end-to-end processing rate. Latency: time from object center to first M: command.

## C. Results

TABLE IV. EXPERIMENTAL RESULTS

| Scenario | Detection | Pickup | Notes |
| --- | --- | --- | --- |
| Clear object, good lighting | [TBD]% | [TBD]% | Baseline |
| Partial occlusion | [TBD]% | [TBD]% | Object partially hidden |
| Low lighting | [TBD]% | [TBD]% | Reduced ambient light |
| Lost detection near gripper | — | [TBD]% | Fallback logic active |

[INSERT FIG. 9: Results visualization (detection %, pickup %)]

Fig. 9. Bar chart or line plot of detection success and pickup success rates across scenarios.

TABLE V. PERFORMANCE METRICS

| Metric | Value |
| --- | --- |
| YOLO inference (416×416) | ~50–80 ms per frame |
| Stream FPS | 15 target |
| Autonomy loop rate | 10 Hz |
| Total hardware cost | [TBD] (Pi, Arduino, motors, sensors, camera) |

TABLE VI. COST COMPARISON

| System | Approx. Cost | Capabilities |
| --- | --- | --- |
| EcoSweep | [TBD] | Edge AI, detection, pickup, Bluetooth app |
| Commercial (e.g., Roomba) | $300–900 | Vacuum, mapping; no discrete pickup |
| Research-grade manipulator | $5,000+ | Full manipulation; cloud or high-end compute |

Key findings: YOLO inference achieves ~50–80 ms per frame at 416×416. Stream FPS meets 15 target. Detection is robust for bottles and cups in clear view; bbox-area stages work well when centered; lost-detection fallback succeeds when the object briefly leaves the frame near contact. Manual override via the app allows recovery from stuck states.

# VII. DISCUSSION

## A. Why the Approach Works

Edge AI removes cloud dependency and latency. Distance stages (FAR, APPROACH_CLOSE, PICKUP) reduce overshoot and enable fine alignment. The stability check (5 frames) reduces spurious pickups. Lost-detection logic compensates for partial visibility when the object is very close.

## B. Limitations

Bbox-area assumes roughly frontal view; extreme viewing angles can misestimate distance. Indoor focus: outdoor lighting and clutter not yet evaluated. Class coverage limited to COCO garbage classes; custom classes would require fine-tuning. Single-object targeting; multi-object prioritization is future work.

## C. Practical Implications

EcoSweep is suitable for indoor (homes, offices, hospitals), outdoor (beaches, wetlands, swampy areas), urban drainage (gutters), and sanitation (manhole-adjacent cleanup). The low cost and edge-based operation make it feasible for resource-constrained environments and pilot deployments.

## D. Extended Applications

The modular platform is extensible to beach cleaning, gutter clearance, swampy-area cleanup, and manhole inspection. Beach/swamp variants could use all-terrain or amphibious chassis. Gutter variants: compact unit at drainage openings. Manhole: downward-facing camera for pre-entry inspection; long-reach arm for surface cleanup. Figure 10 illustrates the manhole concept.

[INSERT FIG. 10: Concept diagram. Source: docs/diagrams/ecosweep-fig7-manhole-concept.puml]

Fig. 10. EcoSweep extended application: manhole inspection concept with downward camera, long-reach arm, safety sensors.

These extensions do not replace trained workers. They suit surface cleanup, pre-entry inspection, and assisted tasks with human supervision. Benefits include reduced human exposure to disease vectors and hazards, and cost-effective deployment compared to industrial robots.

# VIII. CONCLUSION

This paper presented EcoSweep, a modular edge-AI robotic platform for autonomous garbage detection and collection across diverse environments. The system combines YOLOv8 Nano on Raspberry Pi 4, bounding-box area as a distance proxy, and a five-state autonomy logic. Indoor experiments indicate feasible detection and pickup at low cost. The platform is extensible to beach cleaning, gutter clearance, swampy-area cleanup, and manhole inspection. Future work: depth camera integration; custom garbage dataset fine-tuning; multi-object prioritization; outdoor trials (beaches, gutters, swampy terrains); solar-powered deployment; validation of outdoor and manhole variants.

# REFERENCES

# APPENDIX: FIGURE PLACEHOLDERS

The following figures are placeholders. Replace each with the actual image or render from the referenced source. All PlantUML files are in docs/diagrams/.

Fig. 1: System architecture. Source: docs/diagrams/ecosweep-fig1-system-architecture.puml

Fig. 2: Hardware assembly. Source: Photo of robot prototype

Fig. 3: YOLO detection output. Source: Screenshot from FPV stream

Fig. 4: State machine. Source: docs/diagrams/ecosweep-fig4-state-machine.puml

Fig. 5: Key components. Source: Close-up photos

Fig. 6: Data flow. Source: docs/diagrams/ecosweep-data-flow.puml

Fig. 7: Test environment. Source: Photo of setup

Fig. 8: Mobile app. Source: App screenshots

Fig. 9: Results chart. Source: Bar/line chart

Fig. 10: Manhole concept. Source: docs/diagrams/ecosweep-fig7-manhole-concept.puml

# REFERENCES

[1] [To be filled: Survey on waste management robotics]

[2] [To be filled: Autonomous cleaning robots]

[3] iRobot, "Roomba," https://www.irobot.com/

[4] [To be filled: Cloud-based garbage robot]

[5] [To be filled: RGB-D in robotics]

[6] [To be filled: Industrial waste sorting]

[7] G. Mittal et al., "A Survey on Various Approaches for Waste Classification," [Journal/Conference], Year.

[8] [To be filled: YOLO marine debris]

[9] J. Redmon et al., "You Only Look Once: Unified, Real-Time Object Detection," CVPR, 2016.

[10] Ultralytics, "YOLOv8," https://github.com/ultralytics/ultralytics

[11] [To be filled: YOLO on Raspberry Pi]

[12] [To be filled: Edge AI deployment]

[13] [To be filled: State machines in robotics]

[14] [To be filled: Proportional control]